NPS UNIVERSAL DESIGN and ACCESSIBILITY SCOPING FORM

for OUTDOOR DEVELOPED AREAS

Use for Campgrounds, Picnic Areas, Overlooks, Trailheads, Trails and Beach Access Routes, Fishing and Boating Facilities

| PROJECT & PMIS NO.: | DATE: |
| --- | --- |

Prepared by:

Programs and activities provided at this site: Site Arrival; Amenities; Interpretive Programs; Site Programs. Car Parking; Benches/Seating; Amphitheater; Picnicking. Bus Parking; Restrooms; Brochures/Handouts; Camping – RV Back-in. RV Parking.; Family Restrooms; Audiovisual Programs; Camping RV – Pull-through. Overflow/Overnight Parking; Changing Rooms/Showers; Exhibits; Camping –Tent. Employee Parking; Trash/Recycling; Waysides; Camping –Group. Fee Collection Station; Rental Lockers; Large Scale Map; Camp Host Sites. Dump Station; Picnic Tables; Wayfinding Signs; Hiking. Alt. Transportation; Shelters; Cultural Landscape; Overlook. Bus/Shuttle Stop; Water Stations; Special Events; Boating. Service Area; Grill /Fire Ring; Kiosks; Swimming. Walks; Bear-proof Food Locker; Ranger-led Walks/Programs; Beach. Bike Paths; Electrical Hook-ups; Self-Guided Walks; Sport Fields/Courts. Other:; Water Hook-ups; Campsite signs/markers; Bicycles/Boat Rentals. Sewer Hook-ups; Other:; Other:. Other:. ### Universal Design: Universal Design is the design of products and environments to be usable by all people, to the greatest extent possible, without the need for adaptation or specialized design. Most simply, Universal Design is human-centered design with all users in mind. The Seven Principles of Universal Design. Project utilizes the seven principles of universal design throughout the design process to provide a facility that is useable by all. Yes No N/A #### Principle 1: Equitable Use. The design is useful and marketable to people with diverse abilities. Is the same means of use provided for all users: identical whenever possible; equivalent when not? #### Principle 2: Flexibility in Use. Does the design accommodate a wide range of individual preferences and abilities? Is a choice of method provided? #### Principle 3: Simple and Intuitive Use. Use of the design is easy to understand, regardless of the user’s experience, knowledge, language skills, or current concentration level. #### Principle 4: Perceptible Information. Does the design communicate necessary information effectively to the user, regardless of ambient conditions or the user's sensory abilities? #### Principle 5: Tolerance for Error. Is there a tolerance for error? Does the design minimize hazards and the adverse consequences of accidental or unintended actions? #### Principle 6: Low Physical Effort. Does the design require low physical effort? Can the design be used efficiently and comfortably and with a minimum of fatigue? #### Principle 7: Size and Space for Approach and Use. Is there appropriate size and space provided for approach, reach, manipulation, and use regardless of user's body size, posture, or mobility. Is there adequate space provided for the use of assistive devices or personal assistance? ## ABAAS Chapter 2: Scoping Requirements ### F208 Parking Spaces Newly constructed parking facilities shall provide accessible parking spaces in accordance with Table F208.2. Where a parking facility provides separate types of parking spaces (car, recreational vehicle, bus, drop off/unloading, etc.), ABAAS Table F208.2 shall apply to each separate type of parking space provided. #### Table F208.2 Parking Spaces: #### Total Number of Parking Spaces Provided in Parking Facility; #### Minimum Number of Required Accessible Parking Spaces (including van spaces); #### Minimum Number of Required van Accessible Parking Spaces; #### Minimum Number of Required RV/Bus Accessible Parking Spaces. #### 1 to 25; #### 1; #### 1; #### 1. #### 26 to 50; #### 2; #### 1; #### 2. #### 51 to 75; #### 3; #### 1; #### 3. #### 76 to 100; #### 4; #### 1; #### 4. #### 101 to 150; #### 5; #### 1; #### 5. #### 151 to 200; #### 6; #### 2; #### 6. #### 201 to 300; #### 7; #### 2; #### 7. #### 301 to 400; #### 8; #### 2; #### 8. #### Total Parking Spaces provided for this project:; #### # of accessible spaces provided:; #### # of van accessible spaces provided:; #### # of RV/bus accessible spaces provided:. Yes No N/A #### Accessible Parking Spaces. F208.2 Minimum number of accessible parking spaces meets Table F208.2 #### F208.2.4 Van Parking Spaces. For every six or fraction of six accessible parking spaces required, at least one is an accessible van parking space complying with ABAAS 502. #### F208.3 Location. Accessible parking spaces are located on the shortest accessible route from parking to an accessible entrance, 200’ or less from entrance. #### F208.3 Oversized Vehicle Location. Accessible oversized parking spaces for RV’s and buses are located on the shortest accessible route from the oversized vehicle parking facility to an accessible entrance. Accessible spaces are 200’ or less from entrance. #### F208.3 Employee Parking Location. Accessible employee parking spaces are located on the shortest accessible route from the employee parking area to an accessible employee entrance. Accessible spaces are 200’ or less from entrance. #### F208.3 Dispersed Locations. Where parking serves more than one accessible entrance or program, accessible parking spaces are dispersed and located on the shortest accessible route (200’ or less) to the accessible entrances or program. ### F219 Assistive Listening Systems Yes No N/A #### F219.2 Required Systems. In each assembly area (amphitheater) where audio amplification is provided or audible communication is integral to the use of the space, an assistive listening system is provided. Number of receivers meets requirements of ABAAS F219. #### F216.10 Assistive Listening Systems. Assembly areas required by ABAAS F219 to provide assistive listening systems have signs informing patrons of the availability of the assistive listening system. ### F221 Assembly Areas and Outdoor Seating Areas/Amphitheaters #### F221.2.1.1 Number of Wheelchair Spaces in Assembly Areas: Number of Seats in theater or seating area (bench length/24”); Minimum Number of Required Wheelchair Spaces; Number of Seats in Exterior Seating Areas (bench length/24”); Minimum Number of Required Wheelchair Spaces. 4 to 25; 1; 4 to 25; 1. 26 to 50; 2; 26 to 50; 2. 51 to 150; 4; 51 to 150; 4. 151 to 300; 5; 151 to 300; 5. 301 to 500; 6; 301 to 500; 6. #### Number of seats in assembly area:; #### # of accessible spaces provided:; #### Number of seats in exterior seating area:; #### # of accessible spaces provided:. Yes No N/A #### F221.1 General. Assembly areas (amphitheaters) provide wheelchair spaces, companion seats, and designated aisle seats complying with ABAAS F221 and ABAAS 802. In addition, lawn seating complies with ABAAS F221.5. #### F221.2.1 Number. Wheelchair spaces provided comply with figure ABAAS F221.2.1.1 #### F221.2.2 Integration. Wheelchair spaces are integrated into the seating plan. Wheelchair spaces cannot be segregated from general seating areas. #### F221.2.3 Lines of Sight and Dispersion. Wheelchair spaces provide lines of sight complying with ABAAS 802.2. Wheelchair spaces provide spectators with choices of seating locations and viewing angles that are substantially equivalent to, or better than, the choices of seating locations and viewing angles available to all other spectators. #### F221.2.3.1 Horizontal Dispersion and F221.2.3.2 Vertical Dispersion. Wheelchair spaces are dispersed horizontally and vertically. Dispersion is not required for assembly areas with 300 or fewer seats if the wheelchair spaces provide viewing angles that are equivalent to, or better than, the average viewing angle provided in the facility. #### F221.3 Companion Seats. At least one companion seat complying with ABAAS 802.3 is provided for each wheelchair space required by ABAAS F221.2.1. #### F221.4 Designated Aisle Seats. At least 5 percent of the total number of aisle seats provided comply with ABAAS 802.4 (folding armrests and identification) and are the aisle seats located closest to accessible routes. #### F221.5 Lawn Seating. Lawn seating areas and exterior overflow seating areas, where fixed seats are not provided, connect to an accessible route. #### F221.1 Exterior Seating Areas. Site seating areas that accommodate 4 or more persons (2 benches or more) provide integrated wheelchair seating complying with Section ABAAS F221. ### F235 Recreational Boating Facilities Yes No N/A #### Accessible Boat Slips and Boarding Piers. Accessible boat slips and boarding piers Are designed on an accessible route from accessible parking. #### F235.2 Boat Slips. Number or accessible boat slips are provided meeting Table F235.2 min. #### F235.2.1 Dispersion. Accessible boat slips are dispersed throughout the various types of boat slips provided. #### F235.3 Boarding Piers at Boat Launch Ramps. Where boarding piers are provided at boat launch ramps, at least 5 percent, but no less than one, of the boarding piers complies with ABAAS 1003.3.2. ### F237 Fishing Piers and Platforms Yes No N/A #### Fishing Piers. Accessible fishing piers and platforms are on an accessible route from accessible parking. #### F237.1 General. Fishing piers and platforms comply with ABAAS 1005 and are designed with 5’ clear width min. for wheelchair turning space. ## Accessibility Guidelines for Outdoor Developed Areas (AGODA) Scoping Requirements ### F244.2 Camping Units Yes No N/A #### F244.2.1 Minimum Number. Camping facilities provide accessible camping units for all unit types in accordance with Table F244.2.1. Where a camping facility provides separate types of camping units for recreational vehicles only, for tent camping only, or for camp shelters only, Table F244.2.1 of the AGODA apply to each separate type of camping unit provided. Table F244.2.1 Camping Units. Total Number of Camping Units Provided in Camping Facility; Minimum Number of Accessible Camping Units Required. 1; 1. 2 to 25; 2. 26 to 50; 3. 51 to 75; 4. 76 to 100; 5. 101 to 150; 7. 151 to 200; 8. 201 and over; 8, plus 2 percent of the number over 200. #### Total Camping Units provided for this project:; #### # of Accessible Camping Units provided for this project:. #### F244.2.3 Dispersion. Accessible camping unit locations provide choices of camping units comparable to, and integrated with, those available to others. #### F244.3 Outdoor Constructed Features. Where provided within a camping facility or accessible camping unit, outdoor constructed features (picnic tables, grills, water hydrants, hookups, trash and recycling area, etc.) comply with AGODA 1011. ### F245 Picnic Facilities. Picnic facilities provide accessible picnic units in accordance with AGODA F245.2. 1.2, F245.2. 1.2, and F245.2.3 #### Total Picnic Units provided for this project:; #### # of Accessible Picnic Units provided for this project:. Yes No N/A #### F245.2.1.1 Picnic Facilities with Two or Fewer Picnic Units. Where picnic facilities contain two or fewer picnic units, each picnic unit are accessible. #### F245.2.1.2 Picnic Facilities with More Than Two Picnic Units. Where picnic facilities contain more than two picnic units, at least 20 percent but not less than two of the picnic units are accessible. #### F245.2.3 Dispersion. Accessible picnic units provide choices of picnic units comparable to, and integrated with, those available to others #### F245.2.4.1 Outdoor Constructed Features. Where provided within a picnic facility or accessible picnic unit, outdoor constructed features (picnic tables, grills, water hydrants, hookups, trash and recycling area, etc.) comply with AGODA 1011. ### F246 Viewing Areas Yes No N/A #### F246.2 Distinct Viewing Locations. Each distinct viewing location within a viewing area complies with AGODA 1015. Viewing areas can provide more than one distinct viewing location. For example, a viewing area can provide a distinct viewing location for observing a mountain range, and another distinct viewing location for observing a river. Distinct viewing locations within a viewing area are designated by signs or other markers. #### F245.2.4.1 Outdoor Constructed Features. Where provided within a viewing area, outdoor constructed features (trash and recycling receptacles, telescopes etc.) comply with AGODA 1011. ### F247 Trails Yes No N/A #### Trails and Trailheads. Trails and trailheads are designed to comply with AGODA 1017. #### F245.2.4.1 Outdoor Constructed Features. Where provided adjacent to trails, outdoor constructed features (trash and recycling receptacles, telescopes etc.) comply with AGODA 1011. ### F248 Beach Access Routes Yes No N/A #### Beach Access Routes. Beach access routes are designed to comply with AGODA 1018. #### F245.2.4.1 Outdoor Constructed Features. Where provided adjacent to beach access routes, outdoor constructed features (trash and recycling receptacles, telescopes etc.) comply with AGODA 1011. ### 1019 Conditions for Exceptions 1019.1 General. The conditions in AGODA 1019 are the basis for using the exceptions to the requirements in Chapter 10 for outdoor constructed features, tent pads and tent platforms, camp shelters, viewing areas, outdoor recreation access routes, trails, and beach access routes. On outdoor recreation access routes, trails, and beach access routes, the exceptions apply only on the portion of the route where the condition applies. The outdoor recreation access route, trail, or beach access route is required to fully comply with the requirements in Chapter 10 of the AGODA at all other portions of the route where the conditions do not apply. Yes No N/A #### This project will use an exception in AGODA 1019.2 for outdoor constructed features, tent pads and tent platforms, camp shelters, viewing areas, outdoor recreation access routes, trails, or beach access routes. #### Compliance is not feasible due to terrain. #### Compliance cannot be accomplished with the prevailing construction practices. #### Compliance would fundamentally alter the function or purpose of the facility or the setting. #### Compliance is precluded by the: #### Endangered Species Act (16 U.S.C. §§ 1531 et seq.); #### National Environmental Policy Act (42 U.S.C. §§ 4321 et seq.); #### National Historic Preservation Act (16 U.S.C. §§ 470 et seq.); #### Wilderness Act (16 U.S.C. §§ 1131 et seq.); or #### Other Federal, State, or local law the purpose of which is to preserve threatened or endangered species; the environment; or archaeological, cultural, historical, or other significant natural features Basis for exception determination: