NPS UNIVERSAL DESIGN and ACCESSIBILITY SCOPING FORM

for ABAAS FACILITIES

Use for facilities, buildings (new and existing), parking and drop-off areas, and sites (walks, ramps, plazas, lawns, etc.)

| PROJECT & PMIS NO.: | DATE: |
| --- | --- |

Prepared by:

Programs and activities provided at this facility (existing and proposed): Site Programs; Amenities; Building Programs; Interpretive Programs. Car Parking; Benches/Seating; Visitor Use Building; Information Desk. Bus Parking; Restrooms; Historic Building; Brochures/Handouts. RV Parking.; Family Restrooms; Maintenance Building; Audiovisual Programs. Employee Parking; Employee Restrooms; Museum; Exhibits. Building Entrance; Public Telephones; Theater; Interactive Exhibits. Drop-off Area; Drinking Fountains; First Aid/ Wellness Room; Large Scale Map. Alt. Transportation; Vending Machines; Information desk; Tactile Map. Bus/Shuttle Stop; Concessions - Food; Visitor Lodging; Guided Tours. Service Area; Concessions - Ticketing; Employee Housing; Educational Programs. Walks; Gift Shop; Conference Room(s); Museum Objects. Shelters; Bookstore; Office Space; Waysides. Seating/Gathering Space; Trash/Recycling; Kitchen/Break Room; Special Events. Cultural Landscape; Bicycles – racks/rental; Elevator; Self-Guided Walks. Other:; Other:; Other:; Other:. ### Universal Design: Universal Design is the design of products and environments to be usable by all people, to the greatest extent possible, without the need for adaptation or specialized design. Most simply, Universal Design is human-centered design with all users in mind. The Seven Principles of Universal Design. Project utilizes the seven principles of universal design throughout the design process to provide a facility that is useable by all. Yes No N/A #### Principle 1: Equitable Use. The design is useful and marketable to people with diverse abilities. Is the same means of use provided for all users: identical whenever possible; equivalent when not? #### Principle 2: Flexibility in Use. Does the design accommodate a wide range of individual preferences and abilities? Is a choice of method provided? #### Principle 3: Simple and Intuitive Use. Use of the design is easy to understand, regardless of the user’s experience, knowledge, language skills, or current concentration level. #### Principle 4: Perceptible Information. Does the design communicate necessary information effectively to the user, regardless of ambient conditions or the user's sensory abilities? #### Principle 5: Tolerance for Error. Is there a tolerance for error? Does the design minimize hazards and the adverse consequences of accidental or unintended actions? #### Principle 6: Low Physical Effort. Does the design require low physical effort? Can the design be used efficiently and comfortably and with a minimum of fatigue? #### Principle 7: Size and Space for Approach and Use. Is there appropriate size and space provided for approach, reach, manipulation, and use regardless of user's body size, posture, or mobility. Is there adequate space provided for the use of assistive devices or personal assistance? ### DSC Universal Design Best Practice Requirements (* if requirement cannot be met, describe reason for departure and accommodations provided) Yes No* N/A #### Integrated Pedestrian Routes. Project is designed so that all users follow the same routes through the facility and site. Describe any departure: #### Level Entrance. Project is designed so the primary entrances (visitor and employee) do not have steps. Staired secondary routes can be included on sloped sites. Describe any departure: #### Covered Entry. Project provides a covered entry and roof drains away from entry walk and entrance. Describe any departure: #### Close-in parking and drop-off. Distance from drop-off and closest accessible parking space (car and RV/Bus) to accessible entrance of facility is 200’ or less. Describe any departure: #### Power Assist Entrance Doors. Visitor use buildings provide power assist door openers on main accessible entrances. Describe any departure: #### Accessibility Plans. Provide accessibility plans to identify project universal design and accessibility goals. Include site plan with all accessible parking and accessible routes with slope requirements; floorplans with accessible features, accessible routes, turning space, maneuvering space and reach ranges shown. ## ABAAS Chapter 2: Scoping Requirements ### F202 Existing Buildings and Facilities (Renovations and additions including historic facilities) Yes No N/A #### F202.2.1 Accessible Route. An accessible route is provided from accessible parking spaces (car and bus/RV), accessible drop off; public sidewalks; and public transportation to accessible entrances. #### F202.2.3 Toilet Facilities. Project provides at least one men’s and one women’s accessible toilet facility on an accessible route. #### F202.3.1 Prohibited Reduction in Access. Project does not decrease or have the effect of decreasing the accessibility of a building or facility below the requirements for new construction. #### Entrances to existing facilities. If Project is a historic building, is the accessible route the same as general public route? If not, does the accessible route rejoin the general route quickly and intuitively? #### Exception. The only exceptions used for qualified historic buildings and facilities are one accessible route to one accessible entrance (instead of standard 60% of entrances), at least one accessible floor in a multi-story building, and no less than one toilet room for each sex complying with ABAAS 603 or one unisex toilet room. #### SHPO Concurrence. If any of these exceptions are taken, is there a concurrence memo from the SHPO? ABAAS Advisory F202.5 - These exceptions apply only when the State Historic Preservation Officer agrees that compliance with requirements for the specific element would threaten or destroy the historic significance of the building or facility. ### F203 General Exceptions (facility spaces not required to comply with ABAAS requirements) A project element meets one of these exceptions: Yes No N/A #### F203.4 Raised Areas. Areas raised primarily for purposes of security, life safety, or fire safety, including but not limited to, observation or lookout towers, or fire towers #### F203.5 Limited Access Spaces. Spaces accessed only by ladders, catwalks, crawl spaces, or very narrow passageways. #### F203.6 Machinery Spaces. Spaces frequented only by service personnel for maintenance, repair, or occasional monitoring. Machinery spaces include, but are not limited to, elevator, mechanical, electrical or communications equipment rooms; piping or equipment catwalks; water or sewage treatment pump rooms and stations; electric substations and transformer vaults; and highway and tunnel utility facilities. #### F203.7 Single Occupant Structures. Single occupant structures accessed only by passageways below grade or elevated above standard curb height, including but not limited to, toll booths that are accessed only by underground tunnels. ### F205 Operable Parts Yes No N/A #### Hardware. Door, window, restroom and furnishing hardware, levers, knobs, openers, etc. can be operated with one hand and do not require tight grasping, pinching, or twisting of the wrist; are designed between 15” minimum and 48” maximum above the finished floor. The force required to operate hardware is 5 pounds maximum. #### Operable Parts. Light switches, electrical outlets, appliance controls, window blind controls, etc. are designed between 15” minimum and 48” maximum above the finished floor. The force required to activate operable parts is 5 pounds maximum. ### F206 Accessible Routes Yes No N/A #### F206.2.2 Within a Site. At least one accessible route connects accessible buildings, accessible facilities, accessible elements, and accessible spaces that are on the same site. #### F206.2.3 Multi-Story Buildings and Facilities. At least one accessible route connects each story in multi-story buildings and facilities. #### EXCEPTION used for this project: Where a two story building or facility has one story with an occupant load of five or fewer persons that does not contain public use space, that story shall not be required to be connected to the story above or below. #### EXCEPTION used for this project: Where exceptions for alterations to qualified historic buildings or facilities are permitted by ABAAS F202.5, an accessible route shall not be required to stories located above or below the accessible story. – note – programmatic access is required for all programs provided on all floors #### F206.3 Location. Accessible routes coincide with or are located in the same area as general circulation paths. Where circulation paths are interior, accessible routes are also interior. Advisory F206.3 Location. The accessible route must be in the same area as the general circulation path. This means that circulation paths, such as vehicular ways designed for pedestrian traffic, walks, and unpaved paths that are designed to be routinely used by pedestrians must be accessible or have an accessible route nearby. Additionally, accessible vertical interior circulation must be in the same area as stairs and escalators, not isolated in the back of the facility. #### F206.4 Entrances. At least 60 percent of all public entrances and the employee entrance comply with 404, and are on an accessible route complying with ABAAS 402. ### F206.5 Doors, Doorways and Gates Yes No N/A #### Clear Floor Space on both sides of accessible doors. Accessible doors are designed to have adequate clear floor space meeting ABAAS Figure 404.2.4.1. #### F206.5.1 Entrances. Each entrance to a building or facility complying with ABAAS F206.4 has at least one accessible door, doorway, or gate complying with ABAAS 404. #### F206.5.2 Rooms and Spaces. Within a building or facility, at least one door, doorway, or gate serving each accessible room or space is designed to comply with ABAAS 404. ### Figure 404.2.4.1 Maneuvering Clearances at Manual Swinging Doors and Gates ### F207 Accessible Means of Egress Yes No N/A #### F207.1 General. Means of egress complies with applicable section of the 2009 International Building Code (IBC). The accessible egress route is shown on code plan. #### Areas of Refuge. This project includes areas of refuge complying with 2009 IBC that serve as a part of the accessible means of egress. The accessible egress route is shown on code plan. #### F207.2 Platform Lifts. Standby power is provided for platform lifts that serve as a part of the accessible means of egress. ### F208 Parking Spaces Newly constructed parking facilities shall provide accessible parking spaces in accordance with Table F208.2. Where a parking facility provides separate types of parking spaces (car, recreational vehicle, bus, drop off/unloading, etc.), ABAAS Table F208.2 shall apply to each separate type of parking space provided. Yes No N/A #### Accessible Parking Spaces. F208.2 Minimum number of accessible parking spaces meets Table F208.2 #### F208.2.4 Van Parking Spaces. For every six or fraction of six accessible parking spaces required, at least one is an accessible van parking space complying with ABAAS 502. #### F208.3 Location. Accessible parking spaces are located on the shortest accessible route from parking to an accessible entrance, 200’ or less from entrance. #### F208.3 Oversized Vehicle Location. Accessible oversized parking spaces for RV’s and buses are located on the shortest accessible route from the oversized vehicle parking facility to an accessible entrance. Accessible spaces are 200’ or less from entrance. #### F208.3 Employee Parking Location. Accessible employee parking spaces are located on the shortest accessible route from the employee parking area to an accessible employee entrance. Accessible spaces are 200’ or less from entrance. #### F208.3 Dispersed Locations. Where parking serves more than one accessible entrance or program, accessible parking spaces are dispersed and located on the shortest accessible route (200’ or less) to the accessible entrances or program. #### Table F208.2 Parking Spaces: #### Total Number of Parking Spaces Provided in Parking Facility; #### Minimum Number of Required Accessible Parking Spaces (including van spaces); #### Minimum Number of Required van Accessible Parking Spaces; #### Minimum Number of Required RV/Bus Accessible Parking Spaces. #### 1 to 25; #### 1; #### 1; #### 1. #### 26 to 50; #### 2; #### 1; #### 2. #### 51 to 75; #### 3; #### 1; #### 3. #### 76 to 100; #### 4; #### 1; #### 4. #### 101 to 150; #### 5; #### 1; #### 5. #### 151 to 200; #### 6; #### 2; #### 6. #### 201 to 300; #### 7; #### 2; #### 7. #### 301 to 400; #### 8; #### 2; #### 8. #### Total Parking Spaces provided for this project:; #### # of accessible spaces provided:; #### # of van accessible spaces provided:; #### # of RV/bus accessible spaces provided:. ### F209 Passenger Loading Zones and Bus Stops Passenger loading zones (Drop-off areas) shall be provided in accordance with F209. Yes No N/A #### 503 Passenger Loading Zones. Passenger loading zones (Drop-off areas) meet the requirements of ABAAS 503, and are 200’ or less of primary entrance. ### F211 Drinking Fountains Yes No N/A #### F211.2 Drinking Fountains. If provided; at least one has a spout height of 36” max. for wheelchair users; and at least one with a spout height between 38” and 43” for standing persons. ### F213 Toilet Facilities and Bathing Facilities Yes No N/A #### F213.3.1 Toilet Compartments. Where toilet compartments are provided, at least one accessible toilet compartment complies with ABAAS 604.8.1. In addition, at least one ambulatory accessible compartment complies with 604.8.2 where six or more toilet compartments are provided, or where the combination of urinals and water closets totals six or more fixtures. #### 1109.2.1(IBC) Unisex toilet rooms. In assembly and mercantile occupancies, an accessible unisex toilet room is provided where an aggregate of six or more male and female water closets is required. #### F213.3.3 Urinals. Where more than one urinal is provided, at least one is accessible and meets ABAAS 605. #### F213.3.4 Lavatories. Where lavatories are provided, at least one is accessible and complies with ABAAS 606. #### F213.3.5 Mirrors. Where mirrors are provided, at least one is accessible and complies with ABAAS 603.3. #### F213.3.6 Bathing Facilities. Where bathtubs or showers are provided, at least one accessible bathtub or shower complying with ABAAS 607 or 608 is provided. #### Coat hooks and/or Shelves. Where provided, at least one accessible hook and/or shelf is provided between 40” and 48” above the finish floor in the accessible compartment or room. ### F215 Fire Alarm Systems Yes No N/A #### F215.2 Public and Common Use Areas. Alarms in public use areas and common use areas have permanently installed audible and visible alarms complying with ABAAS 702. #### F215.3 Employee Work Areas. Where employee work areas have audible alarm coverage, the wiring system is designed so that visible alarms can be integrated into the system. #### F215.4 Transient Lodging. Guest rooms with communication features required by ABAAS F224.4 have permanently installed audible and visible alarms complying with ABAAS 702. #### F215.5 Residential Facilities. Accessible residences have alarm systems complying with ABAAS 809.5 and 702. ### F216 Signs Yes No N/A #### F216.2 Designations. Interior and exterior signs identifying permanent rooms and spaces have raised characters and braille complying with ABAAS 703.1, 703.2, and 703.5. Where pictograms are provided, they comply with 703.6 and have text descriptors with raised characters and braille complying with ABAAS 703.2 and 703.5. #### F216.4 Means of Egress. Signs for means of egress comply with ABAAS F216.4. #### F216.4.1 Exit Doors. Doors at exit passageways, exit discharge, and exit stairways are identified by signs with raised characters and braille complying with ABAAS 703.1, 703.2, and 703.5. #### F216.4.2 Areas of Refuge. Signs required by the 2009 IBC to provide instructions in areas of refuge comply with ABAAS 703.5. #### F216.4.3 Directional Signs. Signs required by the 2009 IBC to provide directions to accessible means of egress comply with ABAAS 703.5. #### F216.5 Parking. Accessible parking spaces have signs complying with ABAAS 502.6 #### F216.5 Parking. Exception 1 - Parking area for this project has a total of four or fewer parking spaces, including accessible parking spaces; identification of accessible parking spaces is not required. #### F216.6 Entrances. Where not all entrances are accessible, accessible entrances are identified by the International Symbol of Accessibility (ABAAS 703.7.2.1). Directional signs (ABAAS 703.5) are provided at the non-accessible entrances to the nearest accessible entry. #### F216.10 Assistive Listening Systems. Assembly areas required by ABAAS F219 to provide assistive listening systems have signs informing patrons of the availability of the assistive listening system. ### F219 Assistive Listening Systems Yes No N/A #### F219.2 Required Systems. In each assembly area where audio amplification is provided or audible communication is integral to the use of the space, an assistive listening system is provided. Number of receivers meets requirements of ABAAS F219. ### F221 Assembly Areas and Outdoor Seating Areas Yes No N/A #### F221.1 General. Assembly areas provide wheelchair spaces, companion seats, and designated aisle seats complying with ABAAS F221 and ABAAS 802. In addition, lawn seating shall comply with ABAAS F221.5. #### F221.2.1 Number. Wheelchair spaces provided comply with figure ABAAS F221.2.1.1 #### F221.2.2 Integration. Wheelchair spaces are integrated into the seating plan. Wheelchair spaces cannot be segregated from general seating areas. #### F221.2.3 Lines of Sight and Dispersion. Wheelchair spaces provide lines of sight complying with ABAAS 802.2. Wheelchair spaces provide spectators with choices of seating locations and viewing angles that are substantially equivalent to, or better than, the choices of seating locations and viewing angles available to all other spectators. #### F221.2.3.1 Horizontal Dispersion and F221.2.3.2 Vertical Dispersion. Wheelchair spaces are dispersed horizontally and vertically. Dispersion is not required for assembly areas with 300 or fewer seats if the wheelchair spaces provide viewing angles that are equivalent to, or better than, the average viewing angle provided in the facility. #### F221.3 Companion Seats. At least one companion seat complying with ABAAS 802.3 is provided for each wheelchair space required by ABAAS F221.2.1. #### F221.4 Designated Aisle Seats. At least 5 percent of the total number of aisle seats provided comply with ABAAS 802.4 (folding armrests and identification) and are the aisle seats located closest to accessible routes. #### F221.5 Lawn Seating. Lawn seating areas and exterior overflow seating areas, where fixed seats are not provided, connect to an accessible route. #### F221.1 Exterior Seating Areas. Site seating areas that accommodate 4 or more persons (2 benches or more) provide integrated wheelchair seating complying with Section ABAAS F221. #### F221.2.1.1 Number of Wheelchair Spaces in Assembly Areas: Number of Seats in theater or seating area (bench length/24”); Minimum Number of Required Wheelchair Spaces; Number of Seats in Exterior Seating Areas (bench length/24”); Minimum Number of Required Wheelchair Spaces. 4 to 25; 1; 4 to 25; 1. 26 to 50; 2; 26 to 50; 2. 51 to 150; 4; 51 to 150; 4. 151 to 300; 5; 151 to 300; 5. 301 to 500; 6; 301 to 500; 6. #### Number of seats in assembly area:; #### # of accessible spaces provided:; #### Number of seats in exterior seating area:; #### # of accessible spaces provided:. ### F226 Dining Surfaces and Work Surfaces Yes No N/A #### F226.1 General. Where dining surfaces or work surfaces are provided, at least 5 percent are accessible and comply with ABAAS 902. #### F226.2 Dispersion. Accessible dining surfaces and work surfaces are dispersed throughout the space or facility containing dining surfaces and work surfaces. ### F229 Windows Yes No N/A #### F229.1 General. Where glazed openings are provided in accessible rooms or spaces for operation by occupants, at least one window is designed with accessible operable parts complying with ABAAS (Section 309). In accessible rooms or spaces, each glazed opening required by an administrative authority to be operable shall comply with ABAAS 309. #### Window Coverings. Window coverings, blinds, etc. provided in accessible rooms or spaces are designed to be operable at accessible reach ranges (section 308) less than 48” above floor, allow adequate clear floor space for operation and comply with ABAAS 309. Notes: